Solution of HW2

Problem #1 Consider the pin shown

A pin with size and straightness tolerance specifications

Determine:

- The MMC and LMC sizes of the shaft

- The size of the shaft’s fit boundary

- The amount of bending allowed when the feature is 14.6 mm

- The equivalent size limits for ZGT format

Solution

- MMC size = Fmax = 15 mm		LMC size = Fmin =14 mm

- FBF = (Fmax + T) = (15 + 0.5) = 15.5 mm

- Bending = 0.5 + (15 – 14.6) = 0.9 mm

- F0max = Fmax + T = 15.0 + 0.5 = 15.5 mm

Problem #7.  A rectangular key is to fit a keyway as shown.  The key is desired to have an unconstrained fit to the keyway.

Fit of a key and a keyway

The fit tolerances are specified as shown in the figure.  Determine:

- The MMC and LMC sizes of the key and keyway

- The size of the key and keyway fit boundaries

- The amount of center plane bending allowed when the key is made at 4.95 mm

- The amount of center plane bending allowed when the keyway is made at 5.05 mm

- The equivalent size limits for key and keyway in ZGT format

- When the key and the keyway fit, what is the minimum and maximum possible play in the fit?

Solution

- Key:  MMC size = Fmax = 4.95 mm		LMC size = Fmin = 4.90 mm

Keyway: MMC size = Hmin = 5.05 mm	LMC size = Hmax = 5.12 mm

- FBF = 4.95 + 0.05 = 5.00 mm		FBH = 5.05 – 0.05 = 5 mm

- Bending of key allowed at MMC = 0.05 mm

- Bending of the keyway allowed at MMC = 0.05 mm

- Key: 4.9 – 5.0		Keyway: 5.00 – 5.12

- Pmin = FBH – FBF = 5 – 5 = 0 mm

Pmax = Hmax – Fmin = 5.12 – 4.9 = 0.22 mm