ECE 312 Exam #2

- Let x[n]=d[n]+0.2d[n−2]. Find the 50-point DFT for x[n].

(NOTE: just give the equation for the X[k] values.)

dn«1

dn−2«e−j2q

Therefore,

Xq=1+0.2e−j2q

The 50-point DFT samples Xq at 50 points around the unit circle. That is at

qk=2pk50   ;  k=0,1,2,...,49

Thus, the DFT is given by

Xk=1+0.2e−jqk

- A speech signal is sampled at a 20,000 samples/second rate. A sequence of 1024 samples is used to compute a 1024-point DFT.

- What is the time duration of this 1024 sample sequence?

1024 samples20000 samples/s=51.2ms

- What is the frequency resolution (i.e., what is the frequency difference) between DFT samples?

,,

Df=fsN=200001024=19.53Hz.

- For practicing engineers the DFT is far more useful than the CTFS or the CTFT.

Why?

Both the CTFS and the CTFT require solving an integral. This integral cannot be solved unless the equation for the signal of interest is known.

The DFT can be used to help create the DTFS or the DTFT. The DFT only requires samples of the signal of interest. Samples can always be taken without having to know anything about the signal’s equation.

4. For each of the following find the fundamental period N0.

a)   cosp4n+sinp5n

cos2p8n+sin2p10n

The first signal is periodic with period 8 and the second signal with period 10. The least common multiple of 8 and 10 is N0 = 40.

b)   x(t)=cos(18t) with x(t) sampled at a rate Ts=0.1p seconds.

The fundamental period is T0=2p18=p9.

Then, TsT0=mN0 where m and N0 are integers.

N0=mT0Ts=mp/9p/10=m910.  The smallest integer N0 is given when m=10. Thus, N0 = 9.

5.  Explain the relationship between

a) the DFT and the DTFS

The DFT computes coefficients equal to the DTFS Fourier coefficients to within a scale factor. That is, the DFT computed coefficients are N times the DTFS Fourier coefficients.

b) the DFT and the DTFT

If xn«Xq, then the DFT computes sampled values of Xq. The samples are taken at

qk=2pkN where N is the number of samples.

6.  The DTFS of a periodic sequence x[n] has been computed. The Fourier coefficients are

X0=32X1=−12+j12X2=−12X[3]=−12−j12

Find xn for n=0, 1, 2, 3

xn=k=03XkejΩkn  ;   Ω=2p4k=03Xkjkn

This gives

x0=X[0]+X[1]+X[2]+X[3]=0x1=X[0]+jX[1]−X[2]−jX[4]=1x2=X[0]−X[1]+X[2]−X[3]=2x3=X[0]−jX[1]−X[2]+jX[4]=3

7.   The impulse response of a LTI is hn=(0.93)nun.

Find ÐHq.

Hq=11−0.93e−jq=11−0.93cosq+jsinq

Therefore,

ÐHq=  −tan−1sinq1−0.93cosq