- Which memory chips require running a small program if you want to write to them?

- flash

- DRAM chips are denser than SRAM chips

- TRUE

- What is programmed in an FPGA?

d. all of the above

- Which of the following describes the failure rate () over the lifetime of an electronic system?

d. bathtub curve

- Memory access time refers to

- The amount of time it takes to read valid data

- Which of the following can provide active-high combinational outputs?

- (b) and (cd) only

- What is the full page read burst length for a MT48LC64M8A2 SDRAM?

b.  2K

- Reading 4 memory locations from MT48LC128M4A2 SDRAM with a 100 MHz clock takes how much time? (Assume the CAS-latency = 2.)

b. 60 ns

- During the execute command cycle of a SST28SF040A Flash memory, what data is written to the chip during a sector erase?

- D0H

- What is the complete part number an IDT7164  SRAM in a small outline package?

d.  none of the above

- What is the data setup time of an IDT7164S25 SRAM when conducting a write operation?

c. 13 ns

- You have a combinational logic design with 7 inputs and 4 outputs. The minimum size LUT you could use is

c. 128 X 4

- OTP FPGAs are programmed on powerup using a serial flash memory chip.

b. FALSE

- A microprocessor is interfaced to IDT7164S/LS SRAM. The microprocessor WRITE output is connected to the WE input on the memory chip. The WRITE output is asserted low for at least 11 ns during a write operation. Which version of the SRAM will reliably work with this microprocessor?

e. the SRAM cannot be reliably interfaced to the microprocessor

- A 512K x 16 memory system is built using 64K x 4 memory chips. How many memory chips are required?

c. 32

- How many memory locations are in each CLB of a Xilinx Spartan FPGA?

b.  40

- A system is composed of three subsystems with failure rates of 1 = 200 FITs, 2 = 500 FITs  and 3 = 300 FITs. What is the system MTBF?

- 1,000,000 hours

- Two thousand memory chips are operated for 500 hours. Twelve of them fail. What is the MTBF?

- 83,333 hours

- An eletronic circuit has three memory chips (with  = 175 FITs), a microprocessor (with  = 500 FITs) and two PALs (with  = 250 FITs). What is the reliability after 90,000 hours?

c.   87.2%

- One of your fellow students says “my windows VISTA operating system in my laptop is stored in volatile memory”. Do you agree?

b.  NO