# ECE 271 Homework #6

- Let P and R be two 4-bit numbers interpreted to be in 2s complement format.  Show a schematic for a circuit that computes P+R.  (Use a block diagram for the full-adder (FA) instead of individual logic gates.)

- Repeat problem 1 except compute P-R.

- Do the following conversions

- integer 67 to 8-bit 2s complement

- integer –67 to 8-bit 2s complement

- integer –67 to 8-bit sign-magnitude

- 2C903000 (IEEE 754 standard) to a real number

4.   The number 0.27 is to be represented using integer scaling.

- what is the error (in percent) if the scaling is done with n = 6 bits?

- what is the error (in percent) if the scaling is done with n = 8 bits?