Procedure for generating a JEDEC file for GAL programmer

- From your desktop, open ispLEVER (you will find it under “Programs” and then “Lattice Semiconductor”).

- On the menu-bar, click File, then “New project”

- Give your project a name and choose “Schematic/Verilog HDL” for project type.

- Save your project on the C:\Temp folder.

- Click Save.

- On the menu-bar, click Source, then click “Select New Device”

- From the list under “family” choose ‘GAL Device’.

- Under the device list, choose the part that you wish to program (This should be the GAL22V10D).

- Under package type, choose 24PDIP.

- Click OK.

- With the next series of questions answer in this order: YES, YES, OK.

- Click “Source” on the menu bar and choose “Import.”

- Find your Verilog source file and select it, then click “Open”

- Of the three window panes, locate the top level module in the “Sources in project” window and click it.

- With the module selected, double click the line that says “Synplify Synthesize Verilog File” in the right window pane.

- If a red X appears next to the line item, your Verilog file has an error that needs to be fixed.

- If a yellow exclamation mark or green check appear, the synthesis passed (yellow mark indicates warnings, so you may wish to review your code and correct its problem)

- Back in the “Sources In Project” windowpane, click on the device name, “GAL22V10D”

- Once this has been done, the right window pane becomes “Processes for Current Source.” In that window, double click “JEDEC File”

- The output file will be the name of your project with a ‘.JED’ extension. It will be located in the C:\Temp folder (if you saved your project there).

- Close ispLEVER if the JEDEC file generation was successful.

- From your desktop, double click the “SuperPRO” icon. This starts up the programmer software.

- Once started, click “Device” on the menu-bar and choose “Select”. In the “search” area, type GAL22V10D and double click the first device in the list that matches the part number.

- Next, click “File” on the menu bar and choose “Load”. Locate your JEDEC file (should be in the C:\Temp folder) and open it.

- Once open, click the button on the tool bar labeled, “Prog” This will program the part. Provided no errors have occurred, you’re done!

- Close SuperPRO.

- To get the synthesis report that shows pin assignments and the implementation equations, go to the C:\Temp folder where your JEDEC file is located.

- In the C:\Temp folder, double click the file which is named as your project with a ‘.rpt’ extension. This file was generated by “ispLEVER” software when JEDEC file was generated. You can open this file using “Notepad”.

- Print out this ‘.rpt’ file.