Evolvable and Adaptive Hardware

Executive Overview

Summary

Living organisms are extremely complex yet highly reliable and robust systems that developed    without the aid of deliberate design through the mechanisms of evolution natural selection. The field of Evolvable and Adaptive Hardware (EAH) employs computational algorithms based on natural evolution to control the configuration of mechanical and electronic hardware and associated software.    EAH techniques may be applied in several ways. The first is in the automatic design of highly optimized electronic systems,    successful examples being the routing of VLSI chips and the design of high-performance operational amplifiers. A potentially more valuable application is that of being able to reconfigure systems on-the-fly to adapt to changing environmental conditions or to recover from the effects of hardware faults or damage. EAH techniques hold the promise of revolutionizing the design and implementation of complex systems that must reliably interact with the real world.

What is Evolvable and Adaptive Hardware?

Many complex human-engineered systems exhibit a high degree of fragility, in that they cease to function correctly in response to trivial changes in their environments or minor failures within the system.    Living organisms, on the other hand, can often successfully adapt to major changes in their environments or survive, and recover from even severe damage or injuries. As human-engineered systems become increasingly complex, endowing them with the robustness of living systems becomes a highly desirable design objective.

In the 1970s, computer scientists and artificial intelligence researchers began investigating    techniques for solving optimization problems based on biological genetics and evolution. For many classes of problems these ’Genetic’ and ’Evolutionary’ Algorithms offer many advantages over classical mathematical optimization methods, especially when applied to complex real-world problems. Perhaps the greatest advantage is that these techniques work in situations in which the nature of the problem is not well understood; the keys to successfully    employing genetic and evolutionary solution techniques are being able to unambiguously represent potential solutions and to be able to recognize when one potential solution is superior to another.

Many problems associated with designing electronic communication and control systems are amenable to solution by genetic and evolutionary algorithms.    Hardware systems designed or configured by genetic and evolutionary means is referred to Evolvable and Adaptive Hardware (EAH).

Recent advances in the development and availability of dynamically reconfigurable circuitry, such as FPGAs (Field-Programmable Gate Arrays) and FPAAs (Field-Programmable Analog Arrays) has sparked interest in the practical use of EAH techniques for solving many types of design problems. EAH has seen successful applications in the original design of high-performance op-amps and the routing of VLSI chips. A potentially more exciting set of applications is in re-designing systems on-the-fly in response to hardware failure or changes in external environment. This is a far more demanding ‘design’ problem in that one must determine how to utilize    an existing and possibly only partially functional set of hardware to provide acceptable level of    functionality.

How do Evolvable and Adaptive Hardware Techniques    Work?

EAH techniques mirror natural evolution. When employing EAH to design or reconfigure an electronic circuit, the first step is to define a ‘genome’ that describes potential configurations of the circuits one wishes to ’evolve’. In a living organism, the genome consists of a string of amino acids that can be decoded to synthesize proteins. In an EAH problem this genome is a binary string in which different fields indicate circuit characteristics, such as the values of components and the topologies into which to interconnect them.    A genome represents a potential solution to the design problem of interest.

After one has settled on a suitable genome format to    represent potential solutions, the next step is to create an initial population of randomly-generated genomes. This population represents a ‘gene pool’ of potential design solutions, and depending on the nature of the problem, can range from as few as a    half-dozen to several thousand individual genomes.

Once one has set up the initial population, the evolutionary solution process can proceed. This process consists of repeatedly deriving successive generations of ‘offspring’ from an existing population. The fundamental operations that may be applied to a population are:

- Reproduction

- Mutation

- Crossover

- Selection

Reproduction is when a copy of a genome from one generation is passed along to the next generation unchanged. Reproduction is the fundamental means by which solutions are passed along from one generation to the next.

Mutation is when a copy is made of a genome, but a change is made at random. This has the effect of injecting totally new characteristics into the population. As in the case of natural evolution, both when to create a mutation, and the characteristic to mutate is a probabilistic event.

Crossover is when portions of two ‘parent’ genomes are used to create a unique child genome in the next generation. Crossover is different than mutation in that no fundamentally new ‘genetic material’ is created, merely recombined. Crossover is equivalent to sexual reproduction in living organisms.

Selection is the operation of ensuring ‘survival of the fittest’.    While one can perform a selection operation by simply wiping out the least fit members of a population, many selection operators work probabilistically, and will often permit a number of the weaker members to make it to the next generation in the interest of preserving genetic diversity in the population.    Unlike natural selection, the selection operator in an EAH problem is deliberately chosen to result in genomes with desirable characteristics. In this way running an EAH problem is like breeding animals for specific traits.

By repeatedly applying the preceding operators, the population will tend to contain a higher proportion of increasingly ‘fit’ genomes that provide better and better solutions. In general, the more generations for which    one runs an EAH problem, the higher the quality of the solutions represented by the genomes comprising the population.

Practical Applications

For original design, EAH techniques can provide optimal or near-optimal solutions for very complex problems. For example, EAH techniques have been used to route VLSI chips and to design high-performance operational amplifiers. Both of these problems are traditionally very challenging to human designers, but relatively straightforward to solve using EAH techniques. Recently, EAH techniques have been applied to the problem of original circuit design. One example of this work has been the development of CMOS logic circuits which can successfully function at extremely high temperatures for use in space exploration.

Although researchers have just begun exploring the application of EAH techniques for creating fault-tolerant and adaptive    systems, this work has begun to show promising results. For example, one researcher has developed an EAH-configured neural-network controller for a legged walking robot.    By using the EAH to reconfigure the neural-network controller on the fly, this robot (presently implemented in simulation) can adapt its gait to accommodate damage to its sensors and actuators.

The Next Step

EAH techniques will become an increasingly common part of the system designer’s toolbox -    anyplace where failure is not an option and systems must continue to function regardless of failures or external events.      As a newly-emerging technology, EAH will be used in many applications that are yet unimagined. What can you do to explore how your applications may benefit from the use of evolvable and adaptive hardware techniques?

Our first suggestion would be to send a delegate to the IEEE Workshop on Evolvable and Adaptive Hardware. This conference will bring together scientists and engineers from    academia, government and industry to examine recent advances in the field of EAH and to explore the application of EAH techniques in new problem domains. Tutorials, panel discussions, paper presentations, and informal interaction with leaders in research, government and industry will provide numerous and unequalled opportunities to learn about the state-of-the-art in EAH and how it might be applied to solving your particular problems. This conference will be held in Honolulu Hawaii, April 1-5, 2007.

More information of this workshop can be found at http://www.ieee-ssci.org/weah2007/ or you can contact Dr. Garrison Greenwood at xxx@pdx.edu