ME 314 – HW#1

Read

- Section 8-1

- Section 8-2 (no need to read the derivation formulas)

  - Equations 8-1, 8-2, 8-3, 8-4, 8-5, 8-6

  - Skip thread stress analysis

- Section 8-3

- Section 8-4

- Section 8-5 (only the concept and Equation 8-20)

- Section 8-6

Problem 1

A single thread power screw is 25 mm in diameter (outer diameter) and has a thread pitch of 5mm.  Find the thread depth, the thread width, the mean and root diameters, and the lead, provided square threads are used.

Problem 2

A screw clamp similar to the one shown in the figure has a handle with diameter 3/16 in made of cold-drawn AISI 1006 steel. The overall length is 4.25 in (not needed) . The screw is 3/4 in -10 UNC and is 8 in long. Distance A is 3 in. The length of the handle is 4.1875 in (not needed).  The clamp will accommodate parts up to 6 in high.  The screw has a square profile.

- What screw torque will cause the handle to yield?  (26.5 lb-in)

- What is the maximum clamping force (leading to handle yielding) if the collar friction is neglected and if the thread friction is 0.15?  (390 lbs)