ME 314  HW#5

Reading from Chapter 11

- Introduction

- 11-1

- 11-2

- 11-3

- 11-4

- 11-6

- Skip the rest of the chapter

- It is necessary to select a 02-series angular contact bearing for an application.  The following information is give:

- Application Life = 5000 hrs at 480 rpm

- Design Load = 610 lbs

- Application is machinery with light impact: Ka=1.4

- Desired reliability: 90%

- Inner race is rotating and lubrication is adequate

The catalog information is based on 1000 000 cycles and 90% reliability with smooth loading conditions.  Determine:

- The minimum required capacity (Creq)  [~20 KN]

- The bearing size selection (bore size) and the selected bearing capacity (Csel)

- What is the actual reliability of the selected bearing [~91%]

- The following application data is given for a 02-series deep groove ball bearing:

- Radial load = 8 KN

- Axial load = 4 KN

- Application Life = 5000 hours at 900 rpm

- Inner race is rotating and lubrication is adequate

- Required reliability is 95%

- Load-Application factor Ka = 1

The catalog information is based on 1000 000 cycles and 90% reliability with smooth loading conditions.  Determine:

- Selected bearing bore size [85 mm]